ENTERPRISE NETWORK DESIGN

& IMPLEMENTATION

Full Configuration, Screenshots & Troubleshooting Documentation

Cisco Packet Tracer Simulation  |  Enterprise Networking Project

| Degree Programme | B.Tech – Computer Networks & Cloud Security |
| --- | --- |
| University | University of Vocational Technology (UoVT) |
| Simulation Tool | Cisco Packet Tracer 2025 |
| Base Network | 172.16.1.0 |
| Routing Protocol | OSPFv2 – Area 0 |
| Year | 2026 |

Figure 1 — Complete Enterprise Network Topology (Cisco Packet Tracer)

IP Addressing Reference

Department Subnets

| Department | Network | Subnet Mask | Host Range | Broadcast |
| --- | --- | --- | --- | --- |
| Sales & Marketing | 172.16.1.0 | 255.255.255.128/25 | .1 to .126 | 172.16.1.127 |
| HR & Logistics | 172.16.1.128 | 255.255.255.128/25 | .129 to .254 | 172.16.1.255 |
| Finance & Accounts | 172.16.2.0 | 255.255.255.128/25 | .1 to .126 | 172.16.2.127 |
| Admin & PR | 172.16.2.128 | 255.255.255.128/25 | .129 to .254 | 172.16.2.255 |
| ICT | 172.16.3.0 | 255.255.255.128/25 | .1 to .126 | 172.16.3.127 |
| Server Room | 172.16.3.128 | 255.255.255.240/28 | .129 to .142 | 172.16.3.143 |

Figure 2 — IP Addressing Table: First & Second Floor Subnets

Figure 3 — IP Addressing Table: Third Floor, Router-Switch Links & ISP Links

Step 01  —  Network Design Beautification & Hardware Initialization

To establish WAN serial connections between the core network and ISPs, physical hardware expansion was required. Standard Cisco routers do not include serial ports by default — the HWIC-2T module was installed on CORE-R1, CORE-R2, ISP1, and ISP2.

A. HWIC-2T Module Installation (All 4 Routers)

- Open the Physical tab of the router in Packet Tracer.

- Power OFF the router using the power button to prevent hardware damage.

- Drag and drop the HWIC-2T module into an available empty expansion slot.

- Power ON — IOS detects Serial0/2/0 and Serial0/2/1 interfaces automatically.

B. Multilayer Switch Power Supply

- Click MLT-SW1 → Physical tab → drag AC-POWER-SUPPLY into the power bay.

- Cisco IOS boot sequence initiates automatically.

- Repeat for MLT-SW2.

C. Activating Router Interfaces

- Click router → Config tab → select each interface.

- Toggle Port Status to ON — executes no shutdown implicitly.

Final Network Topology

Figure 4 — Final Enterprise Network Topology with all devices active

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| HWIC-2T Module | Provides two high-speed serial WAN ports for DCE/DTE connections, enabling simulated ISP-to-enterprise WAN links. |
| AC-POWER-SUPPLY | Cisco 3650/3850 multilayer switches have modular power designs — explicit power supply insertion is required to boot. |
| Port Status → ON | All Cisco router interfaces are Administratively Down by default. Toggling ON brings the physical layer up. |

Step 02  —  Basic Settings to All Devices + SSH

Baseline security configurations were applied to all access layer switches. SSH v2 was configured on all Layer 3 devices (Routers + Multilayer Switches) to enable encrypted remote management.

SALES-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname SALES-SW

SALES-SW(config)# banner motd #NO Unauthorised Access!!!#

SALES-SW(config)# no ip domain-lookup

SALES-SW(config)# line console 0

SALES-SW(config-line)# password cisco

SALES-SW(config-line)# login

SALES-SW(config-line)# exit

SALES-SW(config)# enable password cisco

SALES-SW(config)# service password-encryption

SALES-SW# write memory

HR-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname HR-SW

HR-SW(config)# banner motd #NO Unauthorised Access!!!#

HR-SW(config)# no ip domain-lookup

HR-SW(config)# line console 0

HR-SW(config-line)# password cisco

HR-SW(config-line)# login

HR-SW(config-line)# exit

HR-SW(config)# enable password cisco

HR-SW(config)# service password-encryption

HR-SW# write memory

FINANCE-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname FINANCE-SW

FINANCE-SW(config)# banner motd #NO Unauthorised Access!!!#

FINANCE-SW(config)# no ip domain-lookup

FINANCE-SW(config)# line console 0

FINANCE-SW(config-line)# password cisco

FINANCE-SW(config-line)# login

FINANCE-SW(config-line)# exit

FINANCE-SW(config)# enable password cisco

FINANCE-SW(config)# service password-encryption

FINANCE-SW# write memory

ADMIN-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname ADMIN-SW

ADMIN-SW(config)# banner motd #NO Unauthorised Access!!!#

ADMIN-SW(config)# no ip domain-lookup

ADMIN-SW(config)# line console 0

ADMIN-SW(config-line)# password cisco

ADMIN-SW(config-line)# login

ADMIN-SW(config-line)# exit

ADMIN-SW(config)# enable password cisco

ADMIN-SW(config)# service password-encryption

ADMIN-SW# write memory

ICT-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname ICT-SW

ICT-SW(config)# banner motd #NO Unauthorised Access!!!#

ICT-SW(config)# no ip domain-lookup

ICT-SW(config)# line console 0

ICT-SW(config-line)# password cisco

ICT-SW(config-line)# login

ICT-SW(config-line)# exit

ICT-SW(config)# enable password cisco

ICT-SW(config)# service password-encryption

ICT-SW# write memory

SERVERROOM-SW — Basic Settings

Switch> enable

Switch# configure terminal

Switch(config)# hostname SERVERROOM-SW

SERVERROOM-SW(config)# banner motd #NO Unauthorised Access!!!#

SERVERROOM-SW(config)# no ip domain-lookup

SERVERROOM-SW(config)# line console 0

SERVERROOM-SW(config-line)# password cisco

SERVERROOM-SW(config-line)# login

SERVERROOM-SW(config-line)# exit

SERVERROOM-SW(config)# enable password cisco

SERVERROOM-SW(config)# service password-encryption

SERVERROOM-SW# write memory

SSH v2 — Routers & Multilayer Switches

Applied to: CORE-R1, CORE-R2, MLT-SW1, MLT-SW2. ISP routers excluded — outside enterprise administrative control.

MLT-SW1 — SSH Configuration

MLT-SW1> enable

MLT-SW1# configure terminal

MLT-SW1(config)# hostname MLT-SW1

MLT-SW1(config)# line console 0

MLT-SW1(config-line)# password cisco

MLT-SW1(config-line)# login

MLT-SW1(config-line)# exit

MLT-SW1(config)# enable password cisco

MLT-SW1(config)# no ip domain-lookup

MLT-SW1(config)# banner motd #NO Unauthorised Access!!!#

MLT-SW1(config)# service password-encryption

MLT-SW1(config)# ip domain-name cisco.net

MLT-SW1(config)# username admin password cisco

MLT-SW1(config)# crypto key generate rsa

! When prompted: enter 1024

MLT-SW1(config)# ip ssh version 2

MLT-SW1(config)# line vty 0 15

MLT-SW1(config-line)# login local

MLT-SW1(config-line)# transport input ssh

MLT-SW1(config-line)# exit

MLT-SW1# write memory

MLT-SW2 — SSH Configuration

MLT-SW2> enable

MLT-SW2# configure terminal

MLT-SW2(config)# hostname MLT-SW2

MLT-SW2(config)# line console 0

MLT-SW2(config-line)# password cisco

MLT-SW2(config-line)# login

MLT-SW2(config-line)# exit

MLT-SW2(config)# enable password cisco

MLT-SW2(config)# no ip domain-lookup

MLT-SW2(config)# banner motd #NO Unauthorised Access!!!#

MLT-SW2(config)# service password-encryption

MLT-SW2(config)# ip domain-name cisco.net

MLT-SW2(config)# username admin password cisco

MLT-SW2(config)# crypto key generate rsa

! When prompted: enter 1024

MLT-SW2(config)# ip ssh version 2

MLT-SW2(config)# line vty 0 15

MLT-SW2(config-line)# login local

MLT-SW2(config-line)# transport input ssh

MLT-SW2(config-line)# exit

MLT-SW2# write memory

CORE-R1 — SSH Configuration

CORE-R1> enable

CORE-R1# configure terminal

CORE-R1(config)# hostname CORE-R1

CORE-R1(config)# line console 0

CORE-R1(config-line)# password cisco

CORE-R1(config-line)# login

CORE-R1(config-line)# exit

CORE-R1(config)# enable password cisco

CORE-R1(config)# no ip domain-lookup

CORE-R1(config)# banner motd #NO Unauthorised Access!!!#

CORE-R1(config)# service password-encryption

CORE-R1(config)# ip domain-name cisco.net

CORE-R1(config)# username admin password cisco

CORE-R1(config)# crypto key generate rsa

! When prompted: enter 1024

CORE-R1(config)# ip ssh version 2

CORE-R1(config)# line vty 0 15

CORE-R1(config-line)# login local

CORE-R1(config-line)# transport input ssh

CORE-R1(config-line)# exit

CORE-R1# write memory

CORE-R2 — SSH Configuration

CORE-R2> enable

CORE-R2# configure terminal

CORE-R2(config)# hostname CORE-R2

CORE-R2(config)# line console 0

CORE-R2(config-line)# password cisco

CORE-R2(config-line)# login

CORE-R2(config-line)# exit

CORE-R2(config)# enable password cisco

CORE-R2(config)# no ip domain-lookup

CORE-R2(config)# banner motd #NO Unauthorised Access!!!#

CORE-R2(config)# service password-encryption

CORE-R2(config)# ip domain-name cisco.net

CORE-R2(config)# username admin password cisco

CORE-R2(config)# crypto key generate rsa

! When prompted: enter 1024

CORE-R2(config)# ip ssh version 2

CORE-R2(config)# line vty 0 15

CORE-R2(config-line)# login local

CORE-R2(config-line)# transport input ssh

CORE-R2(config-line)# exit

CORE-R2# write memory

SSH Verification — Testing Remote Access

! From SysAdmin PC — SSH into CORE-R1

C:\> ssh -l admin 172.16.3.146

Password: cisco

! Expected: Banner appears + CORE-R1> prompt

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| hostname | Modifying the default hostname ensures precise device identification during SSH sessions and network monitoring. |
| banner motd | Provides legal standing in the event of a security breach by warning unauthorized access is prohibited. |
| no ip domain-lookup | Prevents the device from stalling and attempting a DNS broadcast when an admin mistypes a CLI command. |
| service password-encryption | Converts plain-text passwords to encrypted strings (Type-7) in the running-config. |
| crypto key generate rsa 1024 | Generates RSA key pairs required to encrypt SSH sessions. 1024-bit is the minimum for SSHv2. |
| ip ssh version 2 | Enforces SSHv2 only. SSHv1 contains known structural security vulnerabilities. |
| transport input ssh | Blocks Telnet completely on VTY lines — only encrypted SSH connections are permitted. |

Step 03 & 04  —  VLAN Assignment, Trunking & Port Security

VLANs logically segment broadcast domains by department. Trunk ports carry multi-VLAN traffic between switches. Port Security on Finance-SW restricts unauthorized physical connections.

| VLAN ID | Department | Switch | Floor |
| --- | --- | --- | --- |
| VLAN 10 | Sales & Marketing | SALES-SW | 1st Floor |
| VLAN 20 | HR & Logistics | HR-SW | 1st Floor |
| VLAN 30 | Finance & Accounts | FINANCE-SW | 2nd Floor |
| VLAN 40 | Admin & PR | ADMIN-SW | 2nd Floor |
| VLAN 50 | ICT | ICT-SW | 3rd Floor |
| VLAN 60 | Server Room | SERVERROOM-SW | 3rd Floor |
| VLAN 99 | BlackHole (Unused Ports) | All Switches | All Floors |

SALES-SW — VLAN 10 Configuration

SALES-SW> enable

SALES-SW# configure terminal

! Trunk ports to distribution switches

SALES-SW(config)# interface range fa0/1-2

SALES-SW(config-if-range)# switchport mode trunk

SALES-SW(config-if-range)# exit

! Create VLANs

SALES-SW(config)# vlan 10

SALES-SW(config-vlan)# name Sales_Marketing

SALES-SW(config-vlan)# vlan 99

SALES-SW(config-vlan)# name BlackHole

SALES-SW(config-vlan)# exit

! Assign access ports

SALES-SW(config)# interface range fa0/3-24

SALES-SW(config-if-range)# switchport mode access

SALES-SW(config-if-range)# switchport access vlan 10

SALES-SW(config-if-range)# exit

! Isolate unused gigabit ports

SALES-SW(config)# interface range gig0/1-2

SALES-SW(config-if-range)# switchport mode access

SALES-SW(config-if-range)# switchport access vlan 99

SALES-SW(config-if-range)# exit

SALES-SW# write memory

HR-SW — VLAN 20 Configuration

HR-SW> enable

HR-SW# configure terminal

! Trunk ports to distribution switches

HR-SW(config)# interface range fa0/1-2

HR-SW(config-if-range)# switchport mode trunk

HR-SW(config-if-range)# exit

! Create VLANs

HR-SW(config)# vlan 20

HR-SW(config-vlan)# name HR_Logistics

HR-SW(config-vlan)# vlan 99

HR-SW(config-vlan)# name BlackHole

HR-SW(config-vlan)# exit

! Assign access ports

HR-SW(config)# interface range fa0/3-24

HR-SW(config-if-range)# switchport mode access

HR-SW(config-if-range)# switchport access vlan 20

HR-SW(config-if-range)# exit

! Isolate unused gigabit ports

HR-SW(config)# interface range gig0/1-2

HR-SW(config-if-range)# switchport mode access

HR-SW(config-if-range)# switchport access vlan 99

HR-SW(config-if-range)# exit

HR-SW# write memory

FINANCE-SW — VLAN 30 Configuration

FINANCE-SW> enable

FINANCE-SW# configure terminal

! Trunk ports to distribution switches

FINANCE-SW(config)# interface range fa0/1-2

FINANCE-SW(config-if-range)# switchport mode trunk

FINANCE-SW(config-if-range)# exit

! Create VLANs

FINANCE-SW(config)# vlan 30

FINANCE-SW(config-vlan)# name Finance_Accounts

FINANCE-SW(config-vlan)# vlan 99

FINANCE-SW(config-vlan)# name BlackHole

FINANCE-SW(config-vlan)# exit

! Assign access ports

FINANCE-SW(config)# interface range fa0/3-24

FINANCE-SW(config-if-range)# switchport mode access

FINANCE-SW(config-if-range)# switchport access vlan 30

FINANCE-SW(config-if-range)# exit

! Isolate unused gigabit ports

FINANCE-SW(config)# interface range gig0/1-2

FINANCE-SW(config-if-range)# switchport mode access

FINANCE-SW(config-if-range)# switchport access vlan 99

FINANCE-SW(config-if-range)# exit

FINANCE-SW# write memory

ADMIN-SW — VLAN 40 Configuration

ADMIN-SW> enable

ADMIN-SW# configure terminal

! Trunk ports to distribution switches

ADMIN-SW(config)# interface range fa0/1-2

ADMIN-SW(config-if-range)# switchport mode trunk

ADMIN-SW(config-if-range)# exit

! Create VLANs

ADMIN-SW(config)# vlan 40

ADMIN-SW(config-vlan)# name Admin_PR

ADMIN-SW(config-vlan)# vlan 99

ADMIN-SW(config-vlan)# name BlackHole

ADMIN-SW(config-vlan)# exit

! Assign access ports

ADMIN-SW(config)# interface range fa0/3-24

ADMIN-SW(config-if-range)# switchport mode access

ADMIN-SW(config-if-range)# switchport access vlan 40

ADMIN-SW(config-if-range)# exit

! Isolate unused gigabit ports

ADMIN-SW(config)# interface range gig0/1-2

ADMIN-SW(config-if-range)# switchport mode access

ADMIN-SW(config-if-range)# switchport access vlan 99

ADMIN-SW(config-if-range)# exit

ADMIN-SW# write memory

ICT-SW — VLAN 50 Configuration

ICT-SW> enable

ICT-SW# configure terminal

! Trunk ports to distribution switches

ICT-SW(config)# interface range fa0/1-2

ICT-SW(config-if-range)# switchport mode trunk

ICT-SW(config-if-range)# exit

! Create VLANs

ICT-SW(config)# vlan 50

ICT-SW(config-vlan)# name ICT_Dept

ICT-SW(config-vlan)# vlan 99

ICT-SW(config-vlan)# name BlackHole

ICT-SW(config-vlan)# exit

! Assign access ports

ICT-SW(config)# interface range fa0/3-24

ICT-SW(config-if-range)# switchport mode access

ICT-SW(config-if-range)# switchport access vlan 50

ICT-SW(config-if-range)# exit

! Isolate unused gigabit ports

ICT-SW(config)# interface range gig0/1-2

ICT-SW(config-if-range)# switchport mode access

ICT-SW(config-if-range)# switchport access vlan 99

ICT-SW(config-if-range)# exit

ICT-SW# write memory

SERVERROOM-SW — VLAN 60 Configuration

SERVERROOM-SW> enable

SERVERROOM-SW# configure terminal

! Trunk ports to distribution switches

SERVERROOM-SW(config)# interface range fa0/1-2

SERVERROOM-SW(config-if-range)# switchport mode trunk

SERVERROOM-SW(config-if-range)# exit

! Create VLANs

SERVERROOM-SW(config)# vlan 60

SERVERROOM-SW(config-vlan)# name Server_Room

SERVERROOM-SW(config-vlan)# vlan 99

SERVERROOM-SW(config-vlan)# name BlackHole

SERVERROOM-SW(config-vlan)# exit

! Assign access ports

SERVERROOM-SW(config)# interface range fa0/3-24

SERVERROOM-SW(config-if-range)# switchport mode access

SERVERROOM-SW(config-if-range)# switchport access vlan 60

SERVERROOM-SW(config-if-range)# exit

! Isolate unused gigabit ports

SERVERROOM-SW(config)# interface range gig0/1-2

SERVERROOM-SW(config-if-range)# switchport mode access

SERVERROOM-SW(config-if-range)# switchport access vlan 99

SERVERROOM-SW(config-if-range)# exit

SERVERROOM-SW# write memory

Distribution Layer Trunking — MLT-SW1 & MLT-SW2

MLT-SW1

MLT-SW1> enable

MLT-SW1# configure terminal

MLT-SW1(config)# interface range gig1/0/3-8

MLT-SW1(config-if-range)# switchport trunk encapsulation dot1q

MLT-SW1(config-if-range)# switchport mode trunk

MLT-SW1(config-if-range)# exit

MLT-SW1(config)# vlan 10

MLT-SW1(config-vlan)# name Sales

MLT-SW1(config-vlan)# vlan 20

MLT-SW1(config-vlan)# name HR

MLT-SW1(config-vlan)# vlan 30

MLT-SW1(config-vlan)# name Finance

MLT-SW1(config-vlan)# vlan 40

MLT-SW1(config-vlan)# name Admin

MLT-SW1(config-vlan)# vlan 50

MLT-SW1(config-vlan)# name ICT

MLT-SW1(config-vlan)# vlan 60

MLT-SW1(config-vlan)# name ServerRoom

MLT-SW1(config-vlan)# exit

MLT-SW1# write memory

MLT-SW2

MLT-SW2> enable

MLT-SW2# configure terminal

MLT-SW2(config)# interface range gig1/0/3-8

MLT-SW2(config-if-range)# switchport trunk encapsulation dot1q

MLT-SW2(config-if-range)# switchport mode trunk

MLT-SW2(config-if-range)# exit

MLT-SW2(config)# vlan 10

MLT-SW2(config-vlan)# name Sales

MLT-SW2(config-vlan)# vlan 20

MLT-SW2(config-vlan)# name HR

MLT-SW2(config-vlan)# vlan 30

MLT-SW2(config-vlan)# name Finance

MLT-SW2(config-vlan)# vlan 40

MLT-SW2(config-vlan)# name Admin

MLT-SW2(config-vlan)# vlan 50

MLT-SW2(config-vlan)# name ICT

MLT-SW2(config-vlan)# vlan 60

MLT-SW2(config-vlan)# name ServerRoom

MLT-SW2(config-vlan)# exit

MLT-SW2# write memory

Port Security — FINANCE-SW

Finance handles sensitive records — restricting each port to one authorized MAC address prevents rogue device connections.

FINANCE-SW> enable

FINANCE-SW# configure terminal

FINANCE-SW(config)# interface range fa0/3-24

FINANCE-SW(config-if-range)# switchport mode access

FINANCE-SW(config-if-range)# switchport port-security

FINANCE-SW(config-if-range)# switchport port-security maximum 1

FINANCE-SW(config-if-range)# switchport port-security mac-address sticky

FINANCE-SW(config-if-range)# switchport port-security violation shutdown

FINANCE-SW(config-if-range)# exit

FINANCE-SW# write memory

! Verification

FINANCE-SW# show port-security

FINANCE-SW# show port-security interface fa0/3

⚠️  Troubleshooting — Port Security 'Disabled' Status

⚠️  Issue: show port-security returned empty output. Interface showed 'Port Security: Disabled'.

FINANCE-SW# show port-security interface fa0/3

Port Security              : Disabled

Port Status                : Secure-down

Violation Mode             : Shutdown

Maximum MAC Addresses      : 1

Root Cause

- Port security requires dual-execution on large interface ranges in Packet Tracer firmware.

- Sticky MAC remains passive until an end-device sends an active Layer 2 frame.

Fix Applied

FINANCE-SW(config)# interface range fa0/3-24

FINANCE-SW(config-if-range)# switchport mode access

FINANCE-SW(config-if-range)# switchport port-security

FINANCE-SW(config-if-range)# exit

FINANCE-SW# write memory

! Send a ping from Finance PC to trigger MAC learning

! Then verify:

FINANCE-SW# show port-security interface fa0/3

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| switchport mode trunk | Carries frames for all VLANs over a single link using IEEE 802.1Q tagging. |
| switchport mode access | Binds a port to a single VLAN — suitable for PCs and printers. |
| vlan 99 BlackHole | Dead-ends all unused ports, preventing rogue hardware from bridging into the network. |
| switchport trunk encapsulation dot1q | Mandatory on Catalyst 3650/3850 before enabling trunk mode. |
| port-security maximum 1 | Allows only one MAC address per port — prevents unauthorized hubs. |
| mac-address sticky | Dynamically learns and permanently locks the first device MAC address. |
| violation shutdown | Moves the port to err-disabled state immediately when an unauthorized device is detected. |

Step 05  —  Subnetting & IP Addressing

Core interfaces on multilayer switches were converted to Layer 3 routed ports. Serial WAN interfaces on core routers received public IP addresses for ISP connectivity.

IP Addressing Reference Screenshots

Figure 5 — Subnetting Table: First & Second Floor

Figure 6 — Subnetting Table: Third Floor, Router-Switch Links & ISP Public IPs

MLT-SW1 — Layer 3 Routed Ports

MLT-SW1> enable

MLT-SW1# configure terminal

MLT-SW1(config)# interface range gig1/0/1-2

MLT-SW1(config-if-range)# no switchport

MLT-SW1(config-if-range)# exit

! Link toward CORE-R1

MLT-SW1(config)# interface gig1/0/1

MLT-SW1(config-if)# ip address 172.16.3.145 255.255.255.252

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

! Link toward CORE-R2

MLT-SW1(config)# interface gig1/0/2

MLT-SW1(config-if)# ip address 172.16.3.153 255.255.255.252

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1# write memory

MLT-SW2 — Layer 3 Routed Ports

MLT-SW2> enable

MLT-SW2# configure terminal

MLT-SW2(config)# interface range gig1/0/1-2

MLT-SW2(config-if-range)# no switchport

MLT-SW2(config-if-range)# exit

! Link toward CORE-R1

MLT-SW2(config)# interface gig1/0/1

MLT-SW2(config-if)# ip address 172.16.3.149 255.255.255.252

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

! Link toward CORE-R2

MLT-SW2(config)# interface gig1/0/2

MLT-SW2(config-if)# ip address 172.16.3.157 255.255.255.252

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2# write memory

CORE-R1 — Interface IP Addressing

CORE-R1> enable

CORE-R1# configure terminal

! LAN to MLT-SW1

CORE-R1(config)# interface gig0/0

CORE-R1(config-if)# ip address 172.16.3.146 255.255.255.252

CORE-R1(config-if)# no shutdown

CORE-R1(config-if)# exit

! LAN to MLT-SW2

CORE-R1(config)# interface gig0/1

CORE-R1(config-if)# ip address 172.16.3.154 255.255.255.252

CORE-R1(config-if)# no shutdown

CORE-R1(config-if)# exit

! WAN to ISP1 (DCE)

CORE-R1(config)# interface serial 0/2/0

CORE-R1(config-if)# clock rate 64000

CORE-R1(config-if)# ip address 195.136.17.1 255.255.255.252

CORE-R1(config-if)# no shutdown

CORE-R1(config-if)# exit

! WAN to ISP2 (DCE)

CORE-R1(config)# interface serial 0/2/1

CORE-R1(config-if)# clock rate 64000

CORE-R1(config-if)# ip address 195.136.17.5 255.255.255.252

CORE-R1(config-if)# no shutdown

CORE-R1(config-if)# exit

CORE-R1# write memory

CORE-R2 — Interface IP Addressing

CORE-R2> enable

CORE-R2# configure terminal

! LAN to MLT-SW1

CORE-R2(config)# interface gig0/0

CORE-R2(config-if)# ip address 172.16.3.150 255.255.255.252

CORE-R2(config-if)# no shutdown

CORE-R2(config-if)# exit

! LAN to MLT-SW2

CORE-R2(config)# interface gig0/1

CORE-R2(config-if)# ip address 172.16.3.158 255.255.255.252

CORE-R2(config-if)# no shutdown

CORE-R2(config-if)# exit

! WAN to ISP1 (DCE)

CORE-R2(config)# interface serial 0/2/0

CORE-R2(config-if)# clock rate 64000

CORE-R2(config-if)# ip address 195.136.17.9 255.255.255.252

CORE-R2(config-if)# no shutdown

CORE-R2(config-if)# exit

! WAN to ISP2 (DCE)

CORE-R2(config)# interface serial 0/2/1

CORE-R2(config-if)# clock rate 64000

CORE-R2(config-if)# ip address 195.136.17.13 255.255.255.252

CORE-R2(config-if)# no shutdown

CORE-R2(config-if)# exit

CORE-R2# write memory

ISP1 & ISP2

ISP1(config)# interface serial 0/3/0

ISP1(config-if)# ip address 195.136.17.2 255.255.255.252

ISP1(config-if)# no shutdown

ISP1(config)# interface serial 0/3/1

ISP1(config-if)# ip address 195.136.17.10 255.255.255.252

ISP1(config-if)# no shutdown

ISP1# write memory

ISP2(config)# interface serial 0/3/0

ISP2(config-if)# ip address 195.136.17.6 255.255.255.252

ISP2(config-if)# no shutdown

ISP2(config)# interface serial 0/3/1

ISP2(config-if)# ip address 195.136.17.14 255.255.255.252

ISP2(config-if)# no shutdown

ISP2# write memory

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| no switchport | Converts a Layer 2 switch port to a Layer 3 routed interface capable of holding an IP address. |
| /30 Subnet | Point-to-point links only need 2 host IPs — /30 is the most efficient allocation. |
| clock rate 64000 | DCE serial interfaces must provide a clock signal. Without it the serial link stays down. |

Step 06  —  Dynamic Routing — OSPFv2

OSPFv2 was deployed as the IGP. All internal and WAN subnets are converged in Area 0 for automatic route discovery, load sharing and redundancy.

MLT-SW1 — OSPF (Router-ID: 2.2.2.2)

MLT-SW1> enable

MLT-SW1# configure terminal

MLT-SW1(config)# ip routing

MLT-SW1(config)# router ospf 10

MLT-SW1(config-router)# router-id 2.2.2.2

MLT-SW1(config-router)# network 172.16.1.0 0.0.0.127 area 0

MLT-SW1(config-router)# network 172.16.1.128 0.0.0.127 area 0

MLT-SW1(config-router)# network 172.16.2.0 0.0.0.127 area 0

MLT-SW1(config-router)# network 172.16.2.128 0.0.0.127 area 0

MLT-SW1(config-router)# network 172.16.3.0 0.0.0.127 area 0

MLT-SW1(config-router)# network 172.16.3.128 0.0.0.15 area 0

MLT-SW1(config-router)# network 172.16.3.144 0.0.0.3 area 0

MLT-SW1(config-router)# network 172.16.3.152 0.0.0.3 area 0

MLT-SW1(config-router)# exit

MLT-SW1(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet1/0/1

MLT-SW1(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet1/0/2 70

MLT-SW1# write memory

MLT-SW2 — OSPF (Router-ID: 1.1.1.1)

MLT-SW2> enable

MLT-SW2# configure terminal

MLT-SW2(config)# ip routing

MLT-SW2(config)# router ospf 10

MLT-SW2(config-router)# router-id 1.1.1.1

MLT-SW2(config-router)# network 172.16.1.0 0.0.0.127 area 0

MLT-SW2(config-router)# network 172.16.1.128 0.0.0.127 area 0

MLT-SW2(config-router)# network 172.16.2.0 0.0.0.127 area 0

MLT-SW2(config-router)# network 172.16.2.128 0.0.0.127 area 0

MLT-SW2(config-router)# network 172.16.3.0 0.0.0.127 area 0

MLT-SW2(config-router)# network 172.16.3.128 0.0.0.15 area 0

MLT-SW2(config-router)# network 172.16.3.148 0.0.0.3 area 0

MLT-SW2(config-router)# network 172.16.3.156 0.0.0.3 area 0

MLT-SW2(config-router)# exit

MLT-SW2(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet1/0/1

MLT-SW2(config)# ip route 0.0.0.0 0.0.0.0 GigabitEthernet1/0/2 70

MLT-SW2# write memory

CORE-R1 — OSPF (Router-ID: 3.3.3.3)

CORE-R1> enable

CORE-R1# configure terminal

CORE-R1(config)# router ospf 10

CORE-R1(config-router)# router-id 3.3.3.3

CORE-R1(config-router)# network 172.16.3.144 0.0.0.3 area 0

CORE-R1(config-router)# network 172.16.3.152 0.0.0.3 area 0

CORE-R1(config-router)# network 195.136.17.0 0.0.0.127 area 0

CORE-R1(config-router)# exit

CORE-R1(config)# ip route 0.0.0.0 0.0.0.0 Serial0/2/0

CORE-R1(config)# ip route 0.0.0.0 0.0.0.0 Serial0/2/1 70

CORE-R1# write memory

CORE-R2 — OSPF (Router-ID: 4.4.4.4)

CORE-R2> enable

CORE-R2# configure terminal

CORE-R2(config)# router ospf 10

CORE-R2(config-router)# router-id 4.4.4.4

CORE-R2(config-router)# network 172.16.3.148 0.0.0.3 area 0

CORE-R2(config-router)# network 172.16.3.156 0.0.0.3 area 0

CORE-R2(config-router)# network 195.136.17.0 0.0.0.127 area 0

CORE-R2(config-router)# exit

CORE-R2(config)# ip route 0.0.0.0 0.0.0.0 Serial0/2/0

CORE-R2(config)# ip route 0.0.0.0 0.0.0.0 Serial0/2/1 70

CORE-R2# write memory

ISP1 & ISP2 — OSPF

ISP1(config)# router ospf 10

ISP1(config-router)# router-id 5.5.5.5

ISP1(config-router)# network 195.136.17.0 0.0.0.3 area 0

ISP1(config-router)# network 195.136.17.8 0.0.0.3 area 0

ISP1# write memory

ISP2(config)# router ospf 10

ISP2(config-router)# router-id 6.6.6.6

ISP2(config-router)# network 195.136.17.4 0.0.0.3 area 0

ISP2(config-router)# network 195.136.17.12 0.0.0.3 area 0

ISP2# write memory

⚠️  Troubleshooting — OSPF Router-ID Not Taking Effect

⚠️  Issue: OSPF neighbors failed to form FULL adjacency. Router-ID was already selected automatically before manual assignment.

! Fix — Force OSPF process to reload

CORE-R1# clear ip ospf process

Reset ALL OSPF processes? [no]: yes

! Verify

CORE-R1# show ip ospf neighbor

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| ip routing | Activates the Layer 3 routing engine on the multilayer switch. |
| router-id X.X.X.X | Assigns a deterministic OSPF identifier, preventing ID changes during interface flapping. |
| network [IP] [wildcard] area 0 | Advertises the network and enables OSPF Hello packets on matching interfaces. |
| ip route 0.0.0.0 0.0.0.0 [iface] 70 | Backup default route with higher AD — used only when primary fails. |
| clear ip ospf process | Forces OSPF to reload router-id and rebuild adjacencies after configuration changes. |

Step 07  —  Static IP Addressing — Server Room

Core enterprise servers are assigned fixed static IPs. Static addressing prevents critical services from experiencing lease alterations associated with DHCP.

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| DHCP Server | 172.16.3.130 | 255.255.255.240 | 172.16.3.129 |
| DNS Server | 172.16.3.131 | 255.255.255.240 | 172.16.3.129 |
| Email Server | 172.16.3.132 | 255.255.255.240 | 172.16.3.129 |
| SysAdmin PC | 172.16.3.133 | 255.255.255.240 | 172.16.3.129 |

Packet Tracer GUI Steps

- Click on server/PC → Desktop tab → IP Configuration.

- Select Static → enter IP, Subnet Mask, Default Gateway, DNS.

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| Static IP (DHCP Server) | The DHCP server must have a fixed address — if it obtained one dynamically, all client lease requests would fail when that lease shifts. |
| Static IP (DNS Server) | All DHCP pools point to 172.16.3.131. Any change to this address would break name resolution across the entire enterprise. |
| Default Gateway 172.16.3.129 | This is the SVI gateway of VLAN 60 — the Layer 3 exit path for all server traffic. |

Step 08  —  DHCP Server Configuration

Five DHCP pools were provisioned — one per department. Server Room uses static addressing so no pool is required for VLAN 60.

| Pool Name | Default GW | DNS Server | Start IP | Subnet Mask | Max Users |
| --- | --- | --- | --- | --- | --- |
| Sales_Pool | 172.16.1.1 | 172.16.3.131 | 172.16.1.6 | 255.255.255.128 | 120 |
| HR_Pool | 172.16.1.129 | 172.16.3.131 | 172.16.1.134 | 255.255.255.128 | 120 |
| Finance_Pool | 172.16.2.1 | 172.16.3.131 | 172.16.2.6 | 255.255.255.128 | 120 |
| Admin_Pool | 172.16.2.129 | 172.16.3.131 | 172.16.2.134 | 255.255.255.128 | 120 |
| ICT_Pool | 172.16.3.1 | 172.16.3.131 | 172.16.3.6 | 255.255.255.128 | 120 |

GUI Steps

- Click DHCP Server → Services → DHCP → toggle ON.

- Enter pool name, gateway, DNS, start IP, subnet mask, max users → click ADD.

- Repeat for all 5 pools.

DNS Server Configuration

- DNS Server → Services → DNS → toggle ON.

- Add:  Name = www.gtech.com  |  Type = A Record  |  Address = 172.16.3.131 → ADD.

DHCP Verification — Department PCs Getting IP

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| Start IP at .6 / .134 | Lower addresses are reserved for network infrastructure. Offsetting prevents overlap with static switch/router assignments. |
| ip helper-address 172.16.3.130 | DHCP Discover messages are broadcasts that cannot cross VLANs. Helper converts them to unicast toward the DHCP server. |
| DNS A Record | Allows enterprise workstations to resolve www.gtech.com internally without querying external DNS servers. |

Step 09  —  Inter-VLAN Routing & DHCP Helper Addresses

SVIs were created on both MLT-SW1 and MLT-SW2 with identical gateway IPs forming an Active-Active redundant distribution matrix.

MLT-SW1 — SVI Configuration

MLT-SW1> enable

MLT-SW1# configure terminal

! ip routing already enabled from OSPF step

MLT-SW1(config)# interface vlan 10

MLT-SW1(config-if)# ip address 172.16.1.1 255.255.255.128

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1(config)# interface vlan 20

MLT-SW1(config-if)# ip address 172.16.1.129 255.255.255.128

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1(config)# interface vlan 30

MLT-SW1(config-if)# ip address 172.16.2.1 255.255.255.128

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1(config)# interface vlan 40

MLT-SW1(config-if)# ip address 172.16.2.129 255.255.255.128

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1(config)# interface vlan 50

MLT-SW1(config-if)# ip address 172.16.3.1 255.255.255.128

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1(config)# interface vlan 60

MLT-SW1(config-if)# ip address 172.16.3.129 255.255.255.240

MLT-SW1(config-if)# ip helper-address 172.16.3.130

MLT-SW1(config-if)# no shutdown

MLT-SW1(config-if)# exit

MLT-SW1# write memory

MLT-SW2 — SVI Configuration

MLT-SW2> enable

MLT-SW2# configure terminal

! ip routing already enabled from OSPF step

MLT-SW2(config)# interface vlan 10

MLT-SW2(config-if)# ip address 172.16.1.1 255.255.255.128

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2(config)# interface vlan 20

MLT-SW2(config-if)# ip address 172.16.1.129 255.255.255.128

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2(config)# interface vlan 30

MLT-SW2(config-if)# ip address 172.16.2.1 255.255.255.128

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2(config)# interface vlan 40

MLT-SW2(config-if)# ip address 172.16.2.129 255.255.255.128

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2(config)# interface vlan 50

MLT-SW2(config-if)# ip address 172.16.3.1 255.255.255.128

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2(config)# interface vlan 60

MLT-SW2(config-if)# ip address 172.16.3.129 255.255.255.240

MLT-SW2(config-if)# ip helper-address 172.16.3.130

MLT-SW2(config-if)# no shutdown

MLT-SW2(config-if)# exit

MLT-SW2# write memory

Inter-VLAN Ping Test

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| interface vlan [ID] | Creates a Switched Virtual Interface — the logical Layer 3 gateway for all hosts in that VLAN. |
| ip helper-address 172.16.3.130 | Forwards DHCP broadcasts as unicast to the centralized server, enabling cross-VLAN DHCP. |
| Active-Active SVI Design | Identical gateway IPs on both switches allow simultaneous traffic processing and failover. |

Step 10  —  Wireless Network Configuration

Independent Access Points deployed per department with isolated SSIDs and WPA2-PSK encryption.

| Department | AP Name | SSID | WPA2-PSK |
| --- | --- | --- | --- |
| Sales | Sales-AP | SALES-AP | Sales-AP123 |
| HR | HR-AP | HR-AP | HR-AP123 |
| Finance | Finance-AP | Finance-AP | Finance-AP123 |
| Admin | Admin-AP | Admin-AP | Admin-AP123 |
| ICT | ICT-AP | ICT-AP | ICT-AP123 |

Access Point Setup

- Click [Dept]-AP → Config tab → Port 1 → enter SSID → enable WPA2-PSK → enter password.

Laptop Wireless Setup

- Power off laptop → remove Ethernet NM-1CE module → insert WPC300N card → power on.

- Desktop → PC Wireless → Connect → select SSID → enter password → Connect.

Wireless DHCP Verification

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| WPC300N Module Swap | Default laptop modules are wired-only. WPC300N provides 2.4GHz wireless radio capability. |
| WPA2-PSK | Enforces AES block-cipher encryption on all wireless traffic, preventing packet sniffing. |
| Separate SSIDs | Each department discovers only its own network, preventing cross-departmental wireless bridging. |

Step 11  —  PAT + Access Control Lists

PAT allows all internal private addresses to share the public serial interface IP. ACLs define which networks are eligible for NAT translation.

CORE-R1 — NAT/PAT Configuration

CORE-R1> enable

CORE-R1# configure terminal

! Step 1: Define NAT-eligible networks

CORE-R1(config)# access-list 1 permit 172.16.1.0 0.0.0.127

CORE-R1(config)# access-list 1 permit 172.16.1.128 0.0.0.127

CORE-R1(config)# access-list 1 permit 172.16.2.0 0.0.0.127

CORE-R1(config)# access-list 1 permit 172.16.2.128 0.0.0.127

CORE-R1(config)# access-list 1 permit 172.16.3.0 0.0.0.127

CORE-R1(config)# access-list 1 permit 172.16.3.128 0.0.0.15

! Step 2: Bind ACL to WAN interfaces with PAT overload

CORE-R1(config)# ip nat inside source list 1 interface serial 0/2/0 overload

CORE-R1(config)# ip nat inside source list 1 interface serial 0/2/1 overload

! Step 3: Mark LAN interfaces

CORE-R1(config)# interface range gig0/0-1

CORE-R1(config-if-range)# ip nat inside

CORE-R1(config-if-range)# exit

! Step 4: Mark WAN interfaces

CORE-R1(config)# interface serial 0/2/0

CORE-R1(config-if)# ip nat outside

CORE-R1(config-if)# exit

CORE-R1(config)# interface serial 0/2/1

CORE-R1(config-if)# ip nat outside

CORE-R1(config-if)# exit

CORE-R1# write memory

CORE-R2 — NAT/PAT Configuration

CORE-R2> enable

CORE-R2# configure terminal

! Step 1: Define NAT-eligible networks

CORE-R2(config)# access-list 1 permit 172.16.1.0 0.0.0.127

CORE-R2(config)# access-list 1 permit 172.16.1.128 0.0.0.127

CORE-R2(config)# access-list 1 permit 172.16.2.0 0.0.0.127

CORE-R2(config)# access-list 1 permit 172.16.2.128 0.0.0.127

CORE-R2(config)# access-list 1 permit 172.16.3.0 0.0.0.127

CORE-R2(config)# access-list 1 permit 172.16.3.128 0.0.0.15

! Step 2: Bind ACL to WAN interfaces with PAT overload

CORE-R2(config)# ip nat inside source list 1 interface serial 0/2/0 overload

CORE-R2(config)# ip nat inside source list 1 interface serial 0/2/1 overload

! Step 3: Mark LAN interfaces

CORE-R2(config)# interface range gig0/0-1

CORE-R2(config-if-range)# ip nat inside

CORE-R2(config-if-range)# exit

! Step 4: Mark WAN interfaces

CORE-R2(config)# interface serial 0/2/0

CORE-R2(config-if)# ip nat outside

CORE-R2(config-if)# exit

CORE-R2(config)# interface serial 0/2/1

CORE-R2(config-if)# ip nat outside

CORE-R2(config-if)# exit

CORE-R2# write memory

⚠️  Troubleshooting — Empty NAT Table & Ping Failures

⚠️  Issue: show ip nat translations returned empty. Pings to ISP returned 'Request timed out'.

Root Cause 1 — Missing NAT Boundary Tags

GigabitEthernet interfaces lacked 'ip nat inside'. Inbound corporate frames never triggered the NAT lookup engine.

Root Cause 2 — Wildcard Mask Mismatch

Overly restrictive ACL wildcards caused traffic from some floors to fall outside the matching boundary and get dropped without translation.

Root Cause 3 — Asymmetric Return Path

ISP routers lacked static reverse routes, causing return traffic to be dropped before reaching department PCs.

Fix Applied

! Fix 1: Re-declare interface boundaries

CORE-R2(config)# interface GigabitEthernet0/0

CORE-R2(config-if)# ip nat inside

CORE-R2(config-if)# interface Serial0/2/1

CORE-R2(config-if)# ip nat outside

! Fix 2: Widen ACL

CORE-R2(config)# no access-list 1

CORE-R2(config)# access-list 1 permit 172.16.0.0 0.0.255.255

! Fix 3: Add reverse route on ISP

ISP1(config)# ip route 172.16.0.0 255.255.0.0 195.136.17.9

! Verify

CORE-R2# show ip nat translations

CORE-R2# clear ip nat translation *

| Command / Action | Technical Purpose & Why Used |
| --- | --- |
| access-list 1 permit [network] | Classifies which source IPs are eligible for NAT translation. |
| ip nat inside source list 1 interface [serial] overload | Enables PAT — all matching hosts share one public IP, tracked by unique port numbers. |
| ip nat inside / outside | Declares which interfaces are internal (LAN) and external (WAN) NAT boundaries. |
| overload | Enables PAT specifically. Without it, one-to-one static NAT would require a unique public IP per host. |
| clear ip nat translation * | Flushes the NAT cache — useful during troubleshooting to verify fresh translations populate correctly. |

Step 12  —  Verification & Testing

Comprehensive end-to-end testing was conducted after all configurations were applied to verify that every implemented technology functions as expected.

End-to-End Connectivity Test

C:\> ping 195.136.17.10 -n 4

Reply from 195.136.17.10: bytes=32 time=16ms TTL=253

Reply from 195.136.17.10: bytes=32 time=27ms TTL=253

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss)

OSPF Neighbor Verification

CORE-R1# show ip ospf neighbor

! All neighbors in FULL state

DHCP Verification

SSH Test

C:\> ssh -l admin 172.16.3.146

Password: cisco

! CORE-R1> prompt + banner shown

Port Security

NAT/PAT Translation Table

Overall Test Summary

| Test | Verification Method | Result |
| --- | --- | --- |
| Inter-VLAN Connectivity | Ping across departments | Pass ✅ |
| DHCP Auto-Assignment | IP auto-assigned to all PCs | Pass ✅ |
| SSH Remote Access | SSH from SysAdmin PC | Pass ✅ |
| OSPF Adjacency | show ip ospf neighbor | Pass ✅ |
| NAT/PAT Translation | show ip nat translations | Pass ✅ |
| Port Security (Finance) | show port-security | Pass ✅ |
| Wireless Connectivity | Laptop/Tablet connect to AP | Pass ✅ |
| Dual ISP Redundancy | Primary/backup route test | Pass ✅ |

Appendix — AI Assistance Disclosure

This project was independently designed and implemented by the student. Claude AI (Anthropic) was used strictly as a learning assistant to understand concepts — not to generate configurations.

| AI Was Used For | Student Did Independently |
| --- | --- |
| Explaining networking concepts (OSPF, NAT, VLANs, SSH) | Full Packet Tracer topology design |
| Clarifying why each command is used | All CLI configurations typed manually |
| Documentation structure guidance | IP subnetting calculations |
| Reviewing logic for deeper understanding | All troubleshooting — identifying root causes & fixes |
| Answering 'what if' questions during learning | Final testing and verification |

Every command in this document was typed manually and verified in Cisco Packet Tracer. All troubleshooting scenarios were real issues encountered and resolved during the project.

B.Tech in Computer Networks & Cloud Security  |  University of Vocational Technology (UoVT)  |  2026